LitterWatch

AI-Powered Litter Detection & Offender Identification

NextGenHack 2026  |  Demo: 2 March 2026

| Problem ID | 17(check karlena) |
| --- | --- |

| Theme | Smart Cities / Environmental Tech / AI & Computer Vision |
| --- | --- |

| Model mAP50 | 0.81 (YOLOv8s fine-tuned on TACO dataset) |
| --- | --- |

| Stack | Python · YOLOv8 · ByteTrack · FastAPI · SQLite · EasyOCR · HTML/CSS/JS |
| --- | --- |

| Team Size | 3 members — ML/CV · Backend · Frontend/PPT |
| --- | --- |

# 1.  Executive Summary

Littering is a persistent urban problem that costs municipalities millions in cleanup annually while degrading public health and the environment. Existing CCTV infrastructure captures incidents on video but provides no automation — footage is reviewed manually, offenders go unidentified, and actionable data is never collected.

LitterWatch is a real-time AI surveillance pipeline that plugs directly into existing CCTV cameras and solves this problem end-to-end. The system automatically:

- Detects litter objects in live video using a custom-trained YOLOv8s model (TACO dataset, mAP50 = 0.81)

- Tracks every trash object across frames with ByteTrack, assigning persistent IDs

- Determines whether litter was abandoned using a 5-state machine: UNKNOWN → CARRYING → SEPARATION → STATIONARY → ALERTED

- Identifies the offender — pedestrian or vehicle — and captures timestamped photo evidence

- Reads Indian license plates from vehicle crops using EasyOCR with position-aware character correction

- Posts every confirmed incident to a FastAPI + SQLite backend with full metadata

- Streams live annotated video to a surveillance dashboard via MJPEG, decoupled from the detection loop

- Displays all data — incidents, trash types, zone heatmaps, 7-day history, repeat offenders — in a real-time browser dashboard

The entire system was built and integrated in under 4 days by a 3-person team, runs on a single laptop with a consumer GPU, and requires no new hardware — it plugs into any existing IP camera via RTSP or USB.

# 2.  Problem Statement

Urban littering is among the most visible and costly environmental challenges facing Indian cities. Despite widespread CCTV coverage across streets, parks, and transit hubs, the existing infrastructure provides almost zero deterrence or enforcement capability:

- Manual review of hours of CCTV footage is impractical — most footage is never watched

- No automated system exists to distinguish accidental drops from deliberate littering

- Vehicle offenders (littering from cars and auto-rickshaws) are almost never identified due to the speed of the action

- Municipalities have no data on which zones accumulate litter fastest or at which times of day littering peaks

- Clean-up crews operate on fixed schedules rather than data-driven dispatch

The result: billions spent annually on street cleaning, largely reactive rather than preventive. LitterWatch turns the existing CCTV network into an active enforcement and analytics platform.

# 3.  Technical Architecture

The system is composed of three layers that communicate over HTTP on localhost, making it trivially deployable on a single machine or distributable across edge nodes and a cloud backend.

| Layer | Technology | Responsibility |
| --- | --- | --- |
| ML Pipeline | Python / OpenCV | Frame capture, detection, tracking, state machine, OCR, evidence saving |
| Backend API | FastAPI / SQLite | Incident storage, stream serving, stats aggregation, vehicle registry |
| Dashboard | HTML / CSS / JS / Chart.js | Real-time stats, live video feed, incident table, charts, zone map |

## 3.1  ML Pipeline  (detect.py)

The ML pipeline is the core of LitterWatch. It runs on the machine connected to each camera and processes video frames in real time. The pipeline is designed to maintain full detection FPS while streaming annotated frames to the dashboard — these two concerns are fully decoupled.

### Frame Processing Loop

Each frame from the video source (file or live camera) is processed through two YOLO inference calls on detection frames, with a configurable skip rate (SKIP_FRAMES) to control computational cost. Trash tracking runs every frame to keep ByteTrack IDs stable.

- Person/vehicle detection: YOLOv8s pretrained on COCO — detects classes 0 (person), 2 (car), 3 (motorcycle), 5 (bus), 7 (truck)

- Trash detection: YOLOv8s fine-tuned on TACO dataset — detects 81 litter categories including bottles, bags, wrappers, cigarettes, and cans

- ByteTrack multi-object tracker maintains persistent track IDs across frames for all trash objects

- Exponential smoothing applied to bounding box coordinates (alpha=0.5) to reduce jitter in drawn boxes

### 5-State Machine

Every tracked trash object is independently managed through a deterministic state machine. This is the key component that distinguishes deliberate littering from objects being carried, and prevents false positives.

| State | Trigger | Description |
| --- | --- | --- |
| UNKNOWN | Object detected | Trash first appears in scene. Looking for nearest suspect. |
| CARRYING | Suspect within 150px | Trash is near or overlapping a person/vehicle — assumed being carried. |
| SEPARATION | Suspect moves away | Trash and suspect have separated. Monitoring for owner return. |
| STATIONARY | No movement for N frames | Trash has stopped moving and owner has not returned. Potential litter. |
| ALERTED | Person gone beyond threshold | Confirmed littering event. Evidence captured, incident posted to backend. |

Two CANCELLED paths prevent false positives:

- SEPARATION → CANCELLED: if the same person (matched by position proximity) returns within CANCEL_DISTANCE (80px) during the separation window

- STATIONARY → CANCELLED: if the owner returns to the object before the final ALERTED trigger

### Evidence Capture

On transition to ALERTED, the pipeline immediately:

- Crops the offender bounding box from the current frame and saves it as a timestamped JPEG (data/snapshots/persons/ or vehicles/)

- Saves the full annotated frame to data/snapshots/full/

- If the offender is a vehicle, passes the crop to the OCR module for license plate reading

- Calls api_client.post_incident() which POSTs the full event payload to the FastAPI backend

### Stream Sender Thread — Decoupled FPS

A critical performance design: the MJPEG frame stream to the dashboard runs in a dedicated background thread with a shared frame buffer. This means:

- The main detection loop writes the latest annotated frame to a shared variable — this takes ~1 microsecond

- The sender thread independently reads the buffer, JPEG-encodes it at quality=60, and POSTs it to /frame/{camera_id}

- If the backend is slow, the sender automatically skips to the latest frame — it never blocks the detection loop

- Result: detection runs at full GPU speed regardless of network conditions or dashboard load

## 3.2  OCR Module  (ocr_module.py)

EasyOCR is used for license plate recognition. Raw OCR output on vehicle crops is post-processed through a multi-stage correction pipeline tuned for Indian number plates.

- Image preprocessing: grayscale → bilateral filter (noise reduction, edge-preserving) → CLAHE contrast enhancement

- EasyOCR reads with an alphanumeric allowlist (A-Z, 0-9) and returns candidates with confidence scores

- Candidates shorter than 4 characters are discarded

- Position-based character correction applied: first 2 chars (state code) must be letters (0→O, 1→I, 8→B), next 2 (RTO code) must be digits (O→0, I→1), last 4 (serial) must be digits

- Final plate validated against Indian format rules: length 6–12, minimum 2 letters and 1 digit

- Results below 35% confidence are rejected

## 3.3  Backend API  (main.py + DBMS.py)

The FastAPI backend serves as the central data store and stream relay. It uses SQLAlchemy ORM with SQLite (upgradeable to PostgreSQL) and exposes a REST API consumed by both the ML pipeline and the dashboard.

### Database Schema

| Table | Key Columns | Purpose |
| --- | --- | --- |
| incidents | id, timestamp, camera_id, trash_type, offender_type, license_plate, image paths, confidence | Every confirmed littering event with full evidence metadata |
| vehicles | license_plate (unique), first_seen, last_seen, incident_count | Deduplicated vehicle registry; incident_count increments on each re-offence |
| trash_log | timestamp, camera_id, trash_type | Raw trash detection counts batched every 10 seconds from detect.py |

### API Endpoints

| Endpoint | Description |
| --- | --- |
| POST /incidents | Receives confirmed littering events from api_client.py. Creates Vehicle record or increments incident_count. |
| GET /incidents | Returns all incidents ordered by timestamp descending. |
| GET /incidents/recent | Returns last N incidents (default 10). Polled every 5s by dashboard. |
| GET /stats | Aggregated stats: total incidents, total trash, person/vehicle counts, breakdown by trash type and camera. |
| GET /stats/history | 7-day daily incident and trash counts formatted for Chart.js bar chart. |
| GET /vehicles | All vehicles ranked by incident_count descending. |
| POST /frame/{camera_id} | Receives JPEG-encoded frame bytes from detect.py sender thread. Stored in memory. |
| GET /stream/{camera_id} | MJPEG stream — multipart/x-mixed-replace. Browser <img> tag connects directly. |
| GET /cameras/active | Returns cameras that posted a frame in the last 30 seconds. |
| POST /trash_log | Receives batched trash count summaries from detect.py every 10 seconds. |
| GET /snapshots/* | Static file serving for offender crop images and full frames. |
| GET / | Serves the dashboard HTML/CSS/JS as a static site. |

## 3.4  Frontend Dashboard  (index.html / app.js / style.css)

The dashboard is deliberately built as plain HTML/CSS/JS — no React, no build step, no npm. It opens directly in any browser and polls the FastAPI backend every 5 seconds. This was a deliberate choice: zero dependency on frontend tooling, zero risk of build failures during the demo.

### Pages

- Dashboard: 4 stat cards (Total Incidents, Total Trash, Person Offenders, Vehicle Offenders), recent incidents table, trash type pie chart (Chart.js), active cameras list, 7-day history bar chart

- Live Stats: per-camera metrics panel, camera-specific incident breakdown

- Live Feed: MJPEG streams displayed as native <img> tags — no JavaScript decoding. Supports multiple cameras simultaneously. '+Add Camera' button for runtime configuration.

- Incidents: full sortable table of all confirmed events with clickable snapshot thumbnails (lightbox viewer)

- Vehicles: repeat offender registry ranked by incident count with first/last seen timestamps

### Features

- Auto-refresh every 5 seconds with toast notification on new incidents

- Lightbox: click any snapshot thumbnail for full-size evidence view

- Connection status indicator (LIVE / CONNECTING) in header

- Brutalist surveillance aesthetic: dark theme, cyan accents, monospace font (JetBrains Mono), CRT scanline overlay

- Sidebar live stats: total events, persons, vehicles, active camera ID

# 4.  Model Performance

## 4.1  Trash Detection Model

The trash detection model is YOLOv8s fine-tuned on the TACO (Trash Annotations in Context) dataset. TACO contains real-world outdoor litter images with annotations across 60 litter categories.

| Metric | Value | Notes |
| --- | --- | --- |
| mAP50 | 0.81 | Mean Average Precision at IoU threshold 0.50 |
| Base model | YOLOv8s | Small variant — fast inference, low memory footprint |
| Training dataset | TACO | Real-world outdoor litter, 60 categories, 1,500+ images |
| Detection threshold | 0.10 (TRASH_CONF) | Low threshold to maximise recall; state machine handles false positives |
| Tracked classes | Bottles, bags, wrappers, cigarette butts, cans, cardboard, food waste, and more | 81 distinct TACO categories |

## 4.2  Person & Vehicle Detection

Standard YOLOv8s pretrained COCO weights. No fine-tuning required — COCO generalises well to the target environment.

| Class | COCO ID | Detection Threshold |
| --- | --- | --- |
| Person | 0 | 0.40 (PERSON_CONF) |
| Car | 2 | 0.40 |
| Motorcycle | 3 | 0.40 |
| Bus | 5 | 0.40 |
| Truck | 7 | 0.40 |

## 4.3  State Machine Parameters

All thresholds are configurable via config.py. These values were tuned empirically on the test video set.

| Parameter | Value | Meaning |
| --- | --- | --- |
| CARRY_DISTANCE | 150 px | Max distance between trash and suspect to consider 'carrying' |
| SEPARATION_FRAMES | 8 frames | Frames separated before transition to STATIONARY |
| STATIONARY_PIXELS | 15 px | Movement below this threshold = object not moving |
| ABANDON_DISTANCE | 200 px | Suspect must be at least this far before ALERTED triggers |
| CANCEL_DISTANCE | 80 px | Owner must return within this radius to cancel the alert |
| OWNER_MATCH_THRESHOLD | 120 px | Max displacement to match returning person to original owner |
| SKIP_FRAMES | 1 | Run full person detection every N frames (1 = every frame) |
| MEMORY_FRAME_COUNT | 30 frames | Frames before an expired track is purged from object_states |

# 5.  System Configuration

## 5.1  config.py — All Tunable Parameters

All system parameters are centralised in config.py for easy adjustment without touching pipeline code.

| Variable | Default | Purpose |
| --- | --- | --- |
| SOURCE | 1 | Video source: integer for webcam index, or path string for video file / RTSP URL |
| CAMERA_ID | CAM_01 | Identifier sent with every incident and frame POST. Set per camera instance. |
| BACKEND_URL | http://localhost:8000 | FastAPI server base URL. Change for cloud deployment. |
| SNAPSHOT_DIR | data\snapshots | Root folder for saved evidence images. |
| PERSON_CONF | 0.40 | Minimum confidence for person/vehicle detections. |
| TRASH_CONF | 0.10 | Minimum confidence for trash detections. Low to maximise recall. |
| VEHICLE_CLASSES | [2,3,5,7] | COCO class IDs treated as vehicles for state machine purposes. |

## 5.2  Running Multiple Cameras

Each camera is a separate detect.py process. Camera ID and source are set via environment variables:

# Terminal 1 — Camera 1

set CAMERA_ID=CAM_01 && set SOURCE=rtsp://192.168.1.10/stream && py detect.py

# Terminal 2 — Camera 2

set CAMERA_ID=CAM_02 && set SOURCE=rtsp://192.168.1.11/stream && py detect.py

The dashboard auto-discovers active cameras via GET /cameras/active (cameras that POSTed a frame in the last 30 seconds). Stream URLs are constructed automatically as http://localhost:8000/stream/{camera_id}.

# 6.  How to Run (Demo Setup)

The full system runs on a single laptop. Three terminals required.

## 6.1  Prerequisites

- Python 3.12+, pip

- pip install ultralytics easyocr fastapi uvicorn sqlalchemy opencv-python requests

- Model weights: ml_pipeline/weights/yolov8s.pt (COCO) and taco_8s_v3.pt (TACO fine-tuned)

- Any video source: webcam (SOURCE=1), video file, or RTSP camera stream

## 6.2  Start Order

- Start the backend:

cd backend && uvicorn main:app --host 0.0.0.0 --port 8000

- Start detection pipeline (one per camera):

cd ml_pipeline && py detect.py

- Open the dashboard:

http://localhost:8000    (or open frontend/index.html directly in Chrome)

For judges to connect remotely during demo: share laptop IP on local WiFi. Dashboard available at http://<laptop-ip>:8000 from any browser on the same network.

# 7.  File Structure

| File / Folder | Purpose |
| --- | --- |
| ml_pipeline/detect.py | Main detection loop: YOLO inference, ByteTrack, state machine, stream sender thread, HUD drawing |
| ml_pipeline/config.py | All tunable parameters: thresholds, camera ID, source, backend URL |
| ml_pipeline/ocr_module.py | EasyOCR license plate reader with preprocessing and Indian plate validation |
| ml_pipeline/api_client.py | HTTP client for posting incidents and querying stats from the backend |
| backend/main.py | FastAPI app: all REST endpoints, MJPEG stream relay, static file serving |
| backend/DBMS.py | SQLAlchemy ORM models: LitterIncident, Vehicle, TrashLog |
| backend/model.py | Pydantic schemas for request/response validation |
| backend/database.py | SQLAlchemy engine and session factory. DB file: backend/main.db |
| frontend/index.html | Single-page dashboard with 5 tabs: Dashboard, Live Stats, Live Feed, Incidents, Vehicles |
| frontend/app.js | All dashboard logic: API polling, Chart.js charts, MJPEG stream management, lightbox |
| frontend/style.css | Surveillance aesthetic: dark theme, cyan accents, CRT scanlines, responsive layout |
| data/snapshots/persons/ | Offender crop images from confirmed pedestrian littering events |
| data/snapshots/vehicles/ | Vehicle crop images from confirmed vehicle littering events |
| data/snapshots/full/ | Full annotated frames at the moment of each confirmed alert |
| ml_pipeline/weights/yolov8s.pt | COCO pretrained weights for person/vehicle detection |
| ml_pipeline/weights/taco_8s_v3.pt | TACO fine-tuned weights for trash detection (mAP50 = 0.81) |

# 8.  Feasibility & Scalability

## 8.1  What Is Already Working

- Full pipeline demonstrated end-to-end on test video footage

- State machine correctly distinguishes carrying from abandonment, with false-positive cancellation verified

- EasyOCR successfully reading Indian license plates from vehicle crops

- FastAPI backend storing incidents, querying stats, serving MJPEG streams

- Dashboard displaying real-time data, live video, charts, and evidence photos

- Multi-camera architecture designed and tested (multiple detect.py processes, one per camera)

- Frame streaming fully decoupled from detection loop — no FPS impact from network speed

## 8.2  Hardware Requirements

- Minimum: any laptop or desktop with a mid-range GPU (tested on NVIDIA RTX 2050)

- For production: NVIDIA Jetson Orin Nano (~$200) per camera node — runs YOLOv8s at 15-20fps on the edge

- No new cameras required — any IP camera with RTSP output, USB webcam, or existing CCTV DVR/NVR with stream output

- Backend can run on a central server or cloud VM once cameras are network-connected

## 8.3  Scaling to City Level

- Database: SQLite → PostgreSQL. Schema already designed for multi-camera, no changes to application code

- Backend: containerise with Docker, deploy on Railway, AWS, or Azure. FastAPI is production-grade and async

- ML pipeline: one detect.py process per camera, running on Jetson edge nodes near each camera

- Dashboard: already serves from FastAPI as static files — fronting with Nginx adds zero-config horizontal scale

- Alert system (post-hackathon): automated fine generation emails to registered vehicle owners via RTO database integration

## 8.4  Post-Hackathon Improvements

- Merge additional datasets (TrashNet, WasteNet, OpenLitterMap) to push mAP50 from 0.81 toward 0.90+

- Try out MoviNet for HAR Model

- YOLOv8x or YOLOv9 for higher accuracy where FPS is less critical (fixed cameras with slower frame rates)

- Automated RTO plate lookup to identify registered vehicle owner for fine issuance

- Mobile app for field officers to view live alerts and incidents

- SMS/email alert integration for immediate notification of littering events to control room

# 9.  Impact & Benefits

## 9.1  For Municipalities & Urban Bodies

- Automated 24/7 monitoring eliminates dependence on manual CCTV review

- Timestamped, geo-tagged photographic evidence is legally admissible for enforcement proceedings

- Zone-based hotspot identification (6-zone grid per camera) directs cleaning crews to highest-need areas

- Repeat offender database enables escalating penalties and targeted enforcement

- 7-day trend data informs staffing and route planning for cleaning services

- Dwell-time tracking shows how long litter remains on streets before being cleaned — a measurable KPI

## 9.2  For the Environment

- Deterrent effect: visible AI cameras reduce littering frequency once enforcement becomes credible

- Faster detection and dispatch reduces litter dwell time, preventing accumulation and entry into storm drains and waterways

- Trash type analytics reveal which categories (plastic bottles, cigarette butts, food packaging) dominate in each zone — informing targeted awareness campaigns

## 9.3  For Existing Infrastructure

- Zero new hardware required — integrates with any existing CCTV system via RTSP

- Open-source stack (Python, FastAPI, YOLOv8, EasyOCR) keeps deployment and licensing costs near zero

- Maximises ROI on existing city surveillance investments that currently provide no analytical value

- Deployable by any municipality IT team — no specialised ML expertise required to operate

# 10.  Research & References

## 10.1  Datasets

- TACO (Trash Annotations in Context) — Pedamonti, J. et al. (2020). 1,500+ real-world outdoor litter images annotated across 60 litter categories. Primary training dataset for the trash detection model. http://tacodataset.org

- COCO (Common Objects in Context) — Lin, T. et al. (2014). 330,000 images across 80 object categories. Used via pretrained YOLOv8s weights for person and vehicle detection.

## 10.2  Key Papers

- SAWN: A Smart Alert and Warning Network for Littering Surveillance — Nature Scientific Reports, January 2025. Combines MoViNet action recognition with YOLOv8 object detection, achieving 99.5% littering detection accuracy. Validates the core YOLOv8 + state-machine approach used in LitterWatch. https://www.nature.com/articles/s41598-024-77118-x

- https://www.researchgate.net/publication/388326795_Real-time_detection_and_monitoring_of_public_littering_behavior_using_deep_learning_for_a_sustainable_environment

- https://www.irejournals.com/formatedpaper/1712601.pdf

- https://www.samvaktijournals.com/system/files/sjrit/2021.02.19/intelligent_garbage_detection_and_alert_system.pdf

- https://www.ijert.org/ai-based-camera-systems-for-roadsidel-itter-detection-and-offender-identification-ijertv15is010642

- ByteTrack: Multi-Object Tracking by Associating Every Detection Box — Zhang, Y. et al., ECCV 2022. The tracking algorithm used for persistent object ID assignment. Enables the state machine to track individual trash items reliably across frames even with occlusion.

- YOLOv8: Ultralytics YOLOv8 — Jocher, G. et al., Ultralytics, 2023. The object detection architecture for both the trash and person/vehicle models.

https://docs.ultralytics.com/models/yolov8/

## 10.3  Tools & Libraries

| Library / Tool | Usage in LitterWatch |
| --- | --- |
| Ultralytics YOLOv8 | Object detection and tracking (person, vehicle, trash) |
| ByteTrack | Multi-object tracking with persistent IDs across frames |
| EasyOCR | License plate text extraction from vehicle crops |
| OpenCV | Frame capture, image preprocessing, JPEG encoding, bounding box drawing |
| FastAPI | REST API backend — endpoints for incidents, stats, streams |
| SQLAlchemy | ORM for SQLite database — LitterIncident, Vehicle, TrashLog tables |
| Chart.js | Interactive charts in dashboard: pie chart (trash types), bar chart (7-day history) |
| JetBrains Mono | Monospace font used in dashboard for surveillance aesthetic |

— End of Document —